APD voltage measurement

When a beam of light of particular wavelength hit the surface of the photodetector electron hole pair generation occurs and the transition of electron hole pair produce electric current and corresponding voltages

Objective:

- To measure the dark voltage form APD430A

- To measure the voltage generated during white light illumination

Components used:

- HP 2000 Tungsten Halogen light source having optical output 4.5mw

- Optical fiber of 600um core diameter

- Neutral density filter ND10B having optical density 1.0 that reduce the 1/10th of the power from light source

- A 200m focal length AC254-200A achromatic lens

- APD430A photodetector having maximum input power is 1mw

Light source

Optical fiber

ND filter

Achromatic lens

l

Photo detector

Layout diagram of experimental optical setup

PCIe 6323 DAQ

APD430A

SCB 68A

Block diagram of electrical connection of experimental setup

DAQmx Create channel, tells the computer where the physical device is connected for analog input measurement

Flow Chart of LabVIEW Program for voltage measurement:

DAQmx Read, read the analog input and collect 1000k or 10k number of samples per channel from the DAQ

DAQmx Stop and Clear task, stop & reset the DAQ for next acquisition

DAQmx Timing, sets the acquisition clock rate to 100 kHz and allow to measure in Continuous Samples mode

Experimental Result:

- The voltage variation is measured with sample rate 100kHz and number of samples 100Hz, the time interval between each data/sample point is 10us and time taken to acquire 1000 sample points is 10ms.

The graph represents variation of voltage with time and different data points, the sudden rise of base line in graph is due to the shinning of white light.

- The voltage variation is measured for 100kHz sample rate and 10k number of sample points, time taken to acquire 10k sample points is 100ms

Future Work:

- To validate the experimental result.

- To capture the intensity vs time graph.

- To synchronize APD and Galvo scanner in the LabVIEW program for intensity measurement.